CAT Connection Cables

between

RF Amplifier ACOM 600S

and

Transceiver

Document Version:	v1.0

Date:			October 17, 2014

Author:		Nikolay Nenov

Table of Contents

1. ACOM 600S to ELECRAFT K3 RS232 CAT Connection Cable ….……………..….…	3

2. ACOM 600S to ELEKRAFT K3 BCD Band Data CAT Connection Cable ……………	4

3. ACOM 600S to ICOM CI-V CAT Connection Cable …………………….………………	5

4. ACOM 600S to KENWOOD TS-480 RS232 CAT Connection Cable …………………	6

5. ACOM 600S to YAESU FT-920 RS232 CAT Connection Cable ………………………	7

6. ACOM 600S to YAESU FT-1000 RS232 CAT Connection Cable ……..………………	8

7. ACOM 600S to YAESU FT-897 BCD Band Data CAT Connection Cable …..…..……	9

8. ACOM 600S to YAESU FT-1000 BCD Band Data CAT Connection Cable ……….….	10

9. ACOM 600S to YAESU FT-817 Analog Band Data CAT Connection Cable …………	11

1. ACOM 600S to ELECRAFT K3 
    RS232 CAT Connection Cable

| Connectors Type |  |  |  |
| --- | --- | --- | --- |
| Connector 1 D-SUB, High Density, 15 Way, Male (Solder View) |  | Connector 2 D-SUB, 9 Way, Male (Solder View) |  |
| Connections |  |  |  |
| Connection From: | Signal Name | Connection To: | Signal Name |
| Connector 1, pin 2 | RxRS (IN) | Connector 2, pin 2 | RXD IN (OUT) |
| Connector 1, pin 3 | TxRS (OUT) | Connector 2, pin 3 | TXD OUT (IN) |
| Connector 1, pin 5 | Ground | Connector 2, pin 5 | Ground |
| Connector 1, body | Cable Shield | Connector 2, body | Cable Shield |
| Cable Connections |  |  |  |
| Connector 1 Connector 2 (Solder View) (Solder View) |  |  |  |

Note 1: Connection cable must be shielded.

Note 2: This Interface cable is applicable with other compatible transceivers

2. ACOM 600S to ELEKRAFT K3 
    BCD Band Data CAT Connection Cable

| Connectors Type |  |  |  |
| --- | --- | --- | --- |
| Connector 1 D-SUB, High Density, 15 Way, Male (Solder View) |  | Connector 2 D-SUB, High Density, 15 Way, Male (Solder View) |  |
| Connections |  |  |  |
| Connection From: | Signal Name | Connection To: | Signal Name |
| Connector 1, pin 5 | Ground | Connector 2, pin 5 | Ground |
| Connector 1, pin 7 | BDATA0 (IN) | Connector 2, pin 13 | BAND0 (OUT)* |
| Connector 1, pin 8 | BDATA1 (IN) | Connector 2, pin 3 | BAND1 (OUT)* |
| Connector 1, pin 9 | BDATA2 (IN) | Connector 2, pin 9 | BAND2 (OUT)* |
| Connector 1, pin 10 | BDATA3 (IN) | Connector 2, pin 14 | BAND3 (OUT)* |
| Connector 1, body | Cable Shield | Connector 2, body | Cable Shield |
| Cable Connections |  |  |  |
| Connector 1 Connector 2 (Solder View) (Solder View) |  |  |  |

Note 1: Connection cable must be shielded.

Note 2: Be careful not to swap the connectors, because they are of the same type

Note 3: This Interface cable is applicable with other compatible transceivers

* 	Earlier K3 may require external Pull-up resistors to 5V on these lines, typically
2,2 ÷ 10 kOhm (before Ser.N.:2370), after Dec.10.2008 Rev B KIO3 Digital Board.

3. ACOM 600S to ICOM 
    CI-V CAT Connection Cable

| Connectors Type |  |  |  |
| --- | --- | --- | --- |
| Connector 1 D-SUB, High Density, 15 Way, Male (Solder View) |  | Connector 2 Phone Jack 3,5 mm, 2 Way, Male (Side View) |  |
| Connections |  |  |  |
| Connection From: | Signal Name | Connection To: | Signal Name |
| Connector 1, pin 1 | RxTTL (IN) | Connector 1, pin 4 | TxTTL (OUT) |
| Connector 1, pin 1 | TxTTL (OUT) | Connector 2, pin 1 | CI-V (IN/OUT) |
| Connector 1, pin 5 | Ground | Connector 2, pin 2 | Ground |
| Connector 1, body | Cable Shield | Connector 2, body | Cable Shield |
| Cable Connections |  |  |  |
| Connector 1 Connector 2 (Solder View) (Solder View) |  |  |  |

Note 1: Connection cable must be shielded.

Note 2: This Interface cable is applicable with other compatible transceivers

4. ACOM 600S to KENWOOD TS-480 
    RS232 CAT Connection Cable

| Connectors Type |  |  |  |
| --- | --- | --- | --- |
| Connector 1 D-SUB, High Density, 15 Way, Male (Solder View) |  | Connector 2 D-SUB, 9 Way, Female (Solder View) |  |
| Connections |  |  |  |
| Connection From: | Signal Name | Connection To: | Signal Name |
| Connector 1, pin 2 | RxRS (IN) | Connector 2, pin 2 | TxD (OUT) |
| Connector 1, pin 3 | TxRS (OUT) | Connector 2, pin 3 | RxD (IN) |
| Connector 2, pin 7 | RTS (OUT) | Connector 2, pin 8 | CTS (IN) |
| Connector 1, pin 5 | Ground | Connector 2, pin 5 | Ground |
| Connector 1, body | Cable Shield | Connector 2, body | Cable Shield |
| Cable Connections |  |  |  |
| Connector 1 Connector 2 (Solder View) (Solder View) |  |  |  |

Note 1: Connection cable must be shielded.

Note 2: This Interface cable is applicable with other compatible transceivers

5. ACOM 600S to YAESU FT-920 
    RS232 CAT Connection Cable

| Connectors Type |  |  |  |
| --- | --- | --- | --- |
| Connector 1 D-SUB, High Density, 15 Way, Male (Solder View) |  | Connector 2 D-SUB, 9 Way, Male (Solder View) |  |
| Connections |  |  |  |
| Connection From: | Signal Name | Connection To: | Signal Name |
| Connector 1, pin 2 | RxRS (IN) | Connector 2, pin 2 | TxD (OUT) |
| Connector 1, pin 3 | TxRS (OUT) | Connector 2, pin 3 | RxD (IN) |
| Connector 1, pin 5 | Ground | Connector 2, pin 5 | Ground |
| Connector 1, body | Cable Shield | Connector 2, body | Cable Shield |
| Cable Connections |  |  |  |
| Connector 1 Connector 2 (Solder View) (Solder View) |  |  |  |

Note 1: Connection cable must be shielded.

Note 2: This Interface cable is applicable with other compatible transceivers

6. ACOM 600S to YAESU FT-1000
    RS232 CAT Connection Cable

| Connectors Type |  |  |  |
| --- | --- | --- | --- |
| Connector 1 D-SUB, High Density, 15 Way, Male (Solder View) |  | Connector 2 D-SUB, 9 Way, Female (Solder View) |  |
| Connections |  |  |  |
| Connection From: | Signal Name | Connection To: | Signal Name |
| Connector 1, pin 2 | RxRS (IN) | Connector 2, pin 2 | TxD (OUT) |
| Connector 1, pin 3 | TxRS (OUT) | Connector 2, pin 3 | RxD (IN) |
| Connector 1, pin 5 | Ground | Connector 2, pin 5 | Ground |
| Connector 1, body | Cable Shield | Connector 2, body | Cable Shield |
| Cable Connections |  |  |  |
| Connector 1 Connector 2 (Solder View) (Solder View) |  |  |  |

Note 1: Connection cable must be shielded.

Note 2: This Interface cable is applicable with other compatible transceivers

7. ACOM 600S to YAESU FT-897
    BCD Band Data CAT Connection Cable

| Connectors Type |  |  |  |
| --- | --- | --- | --- |
| Connector 1 D-SUB, High Density, 15 Way, Male (Solder View) |  | Connector 2 DIN Miniature Jack, 8 Way, Male (Solder View) |  |
| Connections |  |  |  |
| Connection From: | Signal Name | Connection To: | Signal Name |
| Connector 1, pin 7 | BDATA0 (IN) | Connector 2, pin 4 | BAND A (OUT) |
| Connector 1, pin 8 | BDATA1 (IN) | Connector 2, pin 3 | BAND B (OUT) |
| Connector 1, pin 9 | BDATA2 (IN) | Connector 2, pin 8 | BAND C (OUT) |
| Connector 1, pin 10 | BDATA3 (IN) | Connector 2, pin 7 | BAND D (OUT) |
| Connector 1, pin 13 | KEYIN (IN) | Connector 2, pin 1 | TX GND (OUT) |
| Connector 1, pin 14 | KEYOUT (OUT) | Connector 2, pin 6 | TX INH (IN) |
| Connector 2, pin 6* | Tx INH (IN) | Connector 2, pin 2* | +13.8V (OUT) |
| Connector 1, pin 5 | Ground | Connector 2, pin 5 | Ground |
| Connector 1, body | Cable Shield | Connector 2, body | Cable Shield |
| Cable Connections |  |  |  |
| Connector 1 Connector 2 (Solder View) (Solder View) |  |  |  |

Note 1: Connection cable must be shielded.

Note 2: This Interface cable is applicable with other compatible transceivers

* 	pin 2 and pin 6 of connector 2 must be connected through resistor 10 kOhm / 0.125W

8. ACOM 600S to YAESU FT-1000
    BCD Band Data CAT Connection Cable

| Connectors Type |  |  |  |
| --- | --- | --- | --- |
| Connector 1 D-SUB, High Density, 15 Way, Male (Solder View) |  | Connector 2 DIN Jack, 8 Way, Male (Solder View) |  |
| Connections |  |  |  |
| Connection From: | Signal Name | Connection To: | Signal Name |
| Connector 1, pin 7 | BDATA0 (IN) | Connector 2, pin 3 | BAND A (OUT) |
| Connector 1, pin 8 | BDATA1 (IN) | Connector 2, pin 5 | BAND B (OUT) |
| Connector 1, pin 9 | BDATA2 (IN) | Connector 2, pin 8 | BAND C (OUT) |
| Connector 1, pin 10 | BDATA3 (IN) | Connector 2, pin 1 | BAND D (OUT) |
| Connector 1, pin 13 | KEYIN (IN) | Connector 2, pin 4 | TX GND (OUT) |
| Connector 1, pin 14 | KEYOUT (OUT) | Connector 2, pin 7 | TX INH (IN) |
| Connector 2, pin 7* | Tx INH (IN) | Connector 2, pin 2* | +13.8V (OUT) |
| Connector 1, pin 5 | Ground | Connector 2, pin 6 | Ground |
| Connector 1, body | Cable Shield | Connector 2, body | Cable Shield |
| Cable Connections |  |  |  |
| Connector 1 Connector 2 (Solder View) (Solder View) |  |  |  |

Note 1: Connection cable must be shielded.

Note 2: This Interface cable is applicable with other compatible transceivers

* 	pin 2 and pin 7 of connector 2 must be connected through resistor 10 kOhm / 0.125W

9. ACOM 600S to YAESU FT-817
Analog Band Data CAT Connection Cable

| Connectors Type |  |  |  |
| --- | --- | --- | --- |
| Connector 1 D-SUB, High Density, 15 Way, Male (Solder View) |  | Connector 2 DIN Miniature Jack, 8 Way, Male (Solder View) |  |
| Connections |  |  |  |
| Connection From: | Signal Name | Connection To: | Signal Name |
| Connector 1, pin 6 | bdata (IN) | Connector 2, pin 8 | BAND DATA (OUT) |
| Connector 2, pin 7* | Tx INH (IN) | Connector 2, pin 2* | +13.8V (OUT) |
| Connector 1, pin 5 | Ground | Connector 2, pin 6 | Ground |
| Connector 1, body | Cable Shield | Connector 2, body | Cable Shield |
| Cable Connections |  |  |  |
| Connector 1 Connector 2 (Solder View) (Solder View) |  |  |  |

Note 1: Connection cable must be shielded.

Note 2: This Interface cable is applicable with other compatible transceivers

* 	pin 2 and pin 6 of connector 2 must be connected through resistor 10 kOhm / 0.125W